Toolbox Talk Template:

Title:
Dust and Fume Hazards on Construction Sites

Introduction:
Presenter Name and Role:

Purpose of Today’s Toolbox Talk:
In today’s 10-minute session, we’ll cover:

- The risks of dust and fumes on site

- Relevant legal duties and guidance

- The types of dust and fumes you may be exposed to

- How to control exposure

- What each of us needs to do to stay safe

Key Content:

- Why Dust and Fumes Matter
• Dust and fumes can cause long-term health issues like COPD, asthma, and cancer
• Symptoms often appear too late – prevention is key
• Even if you can’t see them, they can still cause permanent harm

- Legal Requirements and Responsibilities
• Health and Safety at Work etc. Act 1974 – requires a safe working environment
• COSHH Regulations 2002 – requires employers to assess and control exposure to harmful substances
• Employers must:
– Provide ventilation and extraction systems
– Supply appropriate respiratory protection (RPE)
– Deliver training and health surveillance

- Types of Dust and Fumes on Site
• Hazardous Dusts:
– Silica – from cutting bricks, concrete, tiles, stone
– Wood dust – from MDF, sawing, sanding
– General site dust – from sweeping, demolition, mixing
• Fumes:
– Welding and cutting metals
– Diesel engine exhaust
– Bitumen work and burning plastics
– Heated chemicals or solvents
• Fumes are often invisible but highly dangerous – formed when solids are heated and then cool into airborne particles

- How to Control Exposure
• Dust Controls:
– Eliminate: use pre-cut or wet-cut materials
– Substitute: choose lower-dust materials
– Engineering controls: LEV, on-tool extraction
– Admin controls: rotate tasks, reduce exposure time
– PPE: wear properly fitted RPE like FFP3 masks
• Fume Controls:
– Use safer materials where possible
– Ensure LEV and site ventilation are in place
– Limit time spent in high-risk areas
– Maintain all equipment to reduce emissions
– Use correct PPE – RPE, gloves, goggles, coveralls

- What You Need to Do
• Follow best practice:
– Use water suppression or extraction tools
– Avoid dry sweeping – use vacuums or wet methods
– Make sure PPE fits correctly and report any damage
– Keep the site tidy and free from dust build-up
• Your responsibilities:
– Use all control measures and PPE as trained
– Attend training sessions
– Report damaged PPE or equipment
– Speak up about symptoms like coughing or breathlessness

3 Key Takeaways

- Dust and fumes can seriously damage your health – even if you can’t see them.

- Using the correct control measures can significantly reduce your risk.

- Protecting yourself is your responsibility too – report issues, wear PPE, and speak up.

Closing Summary
Dust and fume exposure are silent threats on site. They can cause lifelong illness or worse — but they’re preventable. By using the right equipment, following control measures, and taking responsibility, you help protect not just yourself, but your colleagues too. Remember: Dust can kill. Fumes can too. Control them.