Toolbox Talk Template: Working from Step Ladders and Ladders

Introduction

Presenter Name and Role:
Hi, who you are

Purpose of Today’s Toolbox Talk:
In today’s 10 minute session, we’ll cover:

- The safe use of ladders and step ladders.

- Legal requirements and risk assessments.

- Key safety guidelines, maintenance tips, and weather considerations.

Key Content

1. Legal Requirements

- Under the Work at Height Regulations 2005, work at height must be planned, supervised, and conducted safely.

- Employers must ensure ladders are used appropriately and risks are minimised.

- Risk Assessment: Identify the correct ladder for the task and assess hazards.

- Communicate the controls to all users.

2. Types of Ladders

- Step Ladders: Self-supporting and used for light tasks.

- Ladders: Non-self-supporting and require secure placement against a structure.

3. Key Safety Guidelines

Inspect the Ladder Before Use:

- Ensure it’s in good condition (e.g., no cracks, bent rungs, or missing rubber feet).

- Check the ladder is clean and rated for the task.

Correct Positioning:

- Step Ladders: Fully open with the locking mechanism engaged.

- Ladders: Secure on firm, level ground using the 4:1 ratio (1 foot away for every 4 feet of height).

Climbing Safely:

- Face the ladder and maintain three points of contact.

- Use tool belts to keep hands free.

- Avoid overreaching or leaning to one side.

Working from Ladders:

- Keep your centre of gravity between the side rails.

- Avoid twisting or using ladders for tasks requiring excessive movement.

4. Weather Considerations

- Wind: Avoid ladder use in windy conditions.

- Wet/Icy Conditions: Wait for better conditions or ensure stable ground.

5. Ladder Storage and Maintenance

Storage:

- Keep ladders in a dry, secure place.

Maintenance:

- Conduct regular inspections. Replace or repair damaged parts.

- Perform pre-use checks for stiles, feet, rungs, locking mechanisms, platforms, and steps.

6. Training and Supervision

- Training: All employees must know how to use ladders safely and recognise the right equipment for the task.

- Supervision: Supervisors should enforce safe practices.

Key Takeaways

- Inspect ladders before every use.

- Position and secure them correctly.

- Use the appropriate ladder for the task and follow safe climbing procedures.

- Avoid using ladders in adverse weather conditions.

- Store and maintain ladders properly.

Closing Summary

By following these safety guidelines, we can minimise risks and prevent accidents when working at height.

Remember: Safety is everyone’s responsibility.